차량 시뮬레이션 ICD
(Interface Control Document)

## 1. Hardware Interface (USART2)

| Parameter | Value |
| --- | --- |
| MCU | STM32F103RB (Cortex-M3) |
| Peripheral | USART2 |
| TX pin | PA2 |
| RX pin | PA3 |
| Baud Rate | 115200 |
| Data Bits | 8 |
| Stop Bits | 1 |
| Parity | None |
| Flow Control | None |
| Mode | Full-duplex (TX/RX) |
| Driver | STM32 HAL (HAL_UART_Transmit / HAL_UART_Receive) |
| Frame Format | printf → HAL_UART_Transmit(&huart2, ...) |

## 2. TX Protocol (Firmware 🡪 PC)

### SLAM Telemetry – 10 Hz (every 100ms)

데이터 형식 : SLAM,<dist>,<yaw>,<sonar_r>,<sonar_l>,<sonar_f>\r\n

| Field | Type | Format | Unit | Description |
| --- | --- | --- | --- | --- |
| dist | float | %.2f | cm | Cumulative travel distance from encoder |
| yaw | float | %.2f | deg | Accumulated yaw from MPU6050 gyro |
| sonar_r | float | %.1f | cm | Right ultrasonic (HC-SR04 #0, PB0) |
| sonar_l | float | %.1f | cm | Left ultrasonic (HC-SR04 #1, PA4) |
| sonar_f | float | %.1f | cm | Front ultrasonic (HC-SR04 #2, PC11) |

*Bidirectional tracking achieved by syncing 1-ch EXTI ticks with motor direction.

Example:

SLAM,12.35,-45.20,30.5,25.0,15.3\r\n

Special Values:

- 999.0 — Sensor timeout / no echo (treated as open space)

- 0.0 — Sensor read failure (replaced with 999.0 internally)

- Data Filtering – Each data passes through an EMA filter (40% previous value + 60% new value)

Source  :  Core/Src/main.c:240

### 2.2 Fire Alert – One-shot

데이터 형식 : FIRE_ALERT,TEMP:<temp_c>\r\n

| Field | Type | Format | Unit | Description |
| --- | --- | --- | --- | --- |
| temp_c | float | %.1f | °C | Temperature from KY-013 (ADC CH0) |

Formula :  temp_c = (raw_adc * 330.0) / 4095.0

Example :

FIRE_ALERT,TEMP:25.4\r\n

Triggers when : Flame sensor detects target + laser ON + buzzer alert.

(detects HIGH at least twice during 3 polling attempts at 5ms)

Source : Core/Inc/motor_drive.h:169

### 2.3 Crash Alert -- One-shot

데이터 형식 : CRASH\r\n

Triggers when:

- Shock sensor interrupt (PB12, falling edge, 1500ms cooldown)

- MPU6050 accelerometer G-force > 5.0G on X or Y axis

- Front ultrasonic distance ≤ 4.0cm

- Cliff detected: TCRT5000 detects HIGH signal (PC3, PC4)

- Blind Spot detected: Side IR Sensor (500ms cooldowns)

Source: Core/Inc/motor_drive.h:223, Core/Inc/shock_sensor.h:35, Core/Inc/MPU-6050.h:230

### 2.4 CDS Raw Value — 5 Hz (every 200ms)

데이터 형식 : CDS Raw Value: <value>\r\n

| Field | Type | Format | Description |
| --- | --- | --- | --- |
| value | uint32_t | %lu | Raw ADC reading (CH10, 0–4095) |

Threshold: is_dark = (value < 3000)

Example:

CDS Raw Value: 2048\r\n

Source: Core/Inc/env_sensors.h:71

### 2.5 System Boot/Status Messages

| Message | Description |
| --- | --- |
| \n\n--- SLAM & Drive System Started ---\n | Boot complete |
| MPU6050 Init Failed!\n | MPU6050 not detected |
| 자이로 영점 조절 중.가만히 두세요.\n | Gyro calibrating |
| System Ready. Let's Map!\n | All systems ready |

## 3. RX Protocol (PC → Firmware)

Method: Polled with 1ms blocking timeout via:

HAL_UART_Receive(&huart2, &rx_char, 1, 1)

All commands are single-byte characters. Firmware processes one byte per main loop iteration.

### 3.1 Mode & Motion Commands

| Byte | Function | Mode | Description |
| --- | --- | --- | --- |
| z / Z | Start auto | Any → Auto | Begins autonomous exploration |
| x | Stop | Any → Manual | Stops all motion, exits auto mode, sets car_state = 'x' |
| w | Forward | Manual only | Moves forward at current speed |
| a | Left | Manual only | Spins left in place |
| s | Backward | Manual only | Moves backward at current speed |
| d | Right | Manual only | Spins right in place |

Source: Core/Inc/motor_drive.h:203-218

### 3.2 Speed Control

| Byte | Speed |
| --- | --- |
| 1 | 50% (PWM) |
| 2 | 70% (PWM) |
| 3 | 100% (PWM) |

Source: Core/Inc/motor_drive.h:207-209

### 3.3 Reception Behavior

| Aspect | Detail |
| --- | --- |
| Buffer | Single byte (no RX buffer / FIFO) |
| Timeout | 1ms polling (non-blocking fallback) |
| Auto-mode guard | w/a/s/d ignored if auto_mode == 1 |
| ORE handling | Overrun error flag cleared each loop at Drive_System_Update entry |

### 3.4  I2C CLCD Status Display

### 3.4 .1 Hardware Interface & Pin Configuration

| Parameter | Detail |
| --- | --- |
| Interface Type | Software I2C (Bit-banging) |
| I2C Device Address | 0x27 (or 0x3F depending on PCF8574 chip) |
| SDA Pin (Data) | PC13 (Configured as Output Open-Drain, Pull-up) |
| SCL Pin (Clock) | PC14 (Configured as Output Open-Drain, Pull-up) |
| Bus Speed | ~100 kHz (Standard Mode equivalent via software delay) |
| Driver Mode | Non-blocking sequence parsing within the main loop execution |

### 3.4 .2 RX Protocol: PC Command Format to MCU

Format: $LCD,<DIR>,<ALERT>,<SONAR_F>,<SONAR_L>,<SONAR_R>\n

| Field | Type | Format | Description |
| --- | --- | --- | --- |
| $LCD | String | $LCD | Header packet identifier for CLCD control |
| DIR | Char | %c | Current Direction State ('W': Fwd, 'S': Bwd, 'A': Left, 'D': Right, 'X': Stop) |
| ALERT | String | %s | Emergency Alert Type ('NORMAL', 'CRASH', 'FIRE', 'CLIFF') |
| SONAR_F | Int | %d | Front Obstacle Proximity Status (0: Clear, 1: Warning/Close) |
| SONAR_L | Int | %d | Left Obstacle Proximity Status (0: Clear, 1: Warning/Close) |
| SONAR_R | int | %d | Right Obstacle Proximity Status (0: Clear, 1: Warning/Close) |

## 4. Timing Diagram

*Background Hardware : DWT 🡪 CYCCNT (1/64MHz us unit Timer in HC-SR04.h )

Firmware Main Loop (approx 5–10ms per cycle)

├── EnvSensors_Update()         — digital/ADC reads (every 200ms for CDS)

├── Sonar_Scheduler()           — triggers next sonar (25ms interval per sensor)

├── MPU6050_Update()            — gyro polling + yaw integration

├── Drive_System_Update(&huart2)

│   ├── ORE clear

│   ├── UART RX poll (1ms)      — ← PC command may arrive here

│   ├── Collision checks

│   ├── Auto-navigation FSM

│   └── Manual mode FSM

├── Feedback_Update()           — buzzer PWM toggle

└── [if 100ms elapsed]

└── printf("SLAM,...\r\n")  — → PC receives telemetry

- SLAM TX: 10Hz (100ms interval, main.c:230)

- RX poll: Every main loop iteration (~5-10ms latency)

- CDS TX: 5Hz (200ms interval, env_sensors.h:67)

## 5.Error Handling

| Issue | Mitigation |
| --- | --- |
| UART Overrun (ORE) | __HAL_UART_CLEAR_OREFLAG() at each loop iteration |
| No RX data | HAL_UART_Receive returns HAL_TIMEOUT after 1ms — no action taken |
| Invalid byte | Ignored (only known chars processed) |